# Final Year Project – Literature Survey Guide

Proposed Project: IoT-based Predictive Battery Health Monitoring System for Petrol Two-Wheelers

# 1. Objective of the Literature Survey

The objective is to determine how much work has already been done on battery monitoring, battery health estimation, failure prediction and IoT-based monitoring, and to identify the specific research gap addressed by the proposed project.

# 2. Proposed System – Core Idea

The proposed system aims to continuously monitor a conventional 12V lead-acid battery used in petrol motorcycles/scooters and move beyond simple voltage monitoring toward battery health estimation, degradation analysis, remaining useful life (RUL) and/or failure prediction, with IoT/mobile connectivity and preventive-maintenance recommendations.

# 3. Literature Classification Categories

1. Electric Vehicle Battery Monitoring: EV cars, electric motorcycles/scooters, EV battery monitoring and BMS research.

2. Lead-Acid Battery Monitoring: Generic lead-acid applications such as UPS, solar, inverter, telecom and laboratory systems.

3. Automotive Battery Monitoring: 12V batteries in cars, trucks and fleet vehicles; cranking and alternator diagnostics.

4. Motorcycle / Two-Wheeler Battery Monitoring: Petrol motorcycles/scooters and their conventional 12V batteries. This is the closest target category.

5. Battery Health Estimation & Prediction: SOH, SOC, RUL, capacity estimation, degradation prediction, failure prediction and predictive maintenance.

6. IoT-Based Battery Monitoring: ESP32/Arduino/Raspberry Pi, Wi-Fi/Bluetooth, MQTT, cloud platforms, mobile apps and alerts.

7. Commercial Products: Existing battery monitors, diagnostic devices, Bluetooth products and manufacturer solutions.

8. Patents (Optional): Patented battery monitoring, health estimation, vehicle battery diagnostics and prediction systems.

# 4. Recommended Paper Documentation Format

Paper ID:

Title:

Year:

Primary Category:

Secondary Category:

Application:

Battery Type:

Hardware Used:

Sensors Used:

Communication:

IoT:

Mobile App:

Cloud:

Monitoring:

SOH Estimation:

RUL Prediction:

Failure Prediction:

Algorithm / Method:

Strengths:

Limitations:

Comparison with Proposed Solution:

Research Gap:

# 5. Standard Comparison Matrix

| Feature | Existing Paper | Proposed Solution |
| --- | --- | --- |
| Vehicle / Application |  |  |
| Battery Type |  |  |
| BMS |  |  |
| IoT |  |  |
| Mobile App |  |  |
| Cloud |  |  |
| Voltage Monitoring |  |  |
| Current Monitoring |  |  |
| Temperature Monitoring |  |  |
| SOH Estimation |  |  |
| RUL Prediction |  |  |
| Failure Prediction |  |  |
| Alerts |  |  |
| Primary Objective |  |  |

# 6. Search Sources

Google Scholar – https://scholar.google.com

IEEE Xplore – https://ieeexplore.ieee.org

ScienceDirect – https://www.sciencedirect.com

SpringerLink – https://link.springer.com

Semantic Scholar – https://www.semanticscholar.org

ResearchGate – https://www.researchgate.net

Google Patents – https://patents.google.com

# 7. Suggested Search Queries

"motorcycle battery health monitoring"

"motorcycle battery monitoring system"

"two wheeler battery monitoring"

"two wheeler battery health"

"lead acid battery health monitoring vehicle"

"automotive lead acid battery state of health"

"vehicle battery state of health estimation"

"IoT battery monitoring lead acid"

"battery remaining useful life lead acid"

"battery failure prediction lead acid"

"battery degradation prediction lead acid"

"12V lead acid vehicle battery health prediction"

"motorcycle lead acid battery SOH"

"motorcycle lead acid battery RUL"

"two wheeler battery predictive maintenance"

# 8. Papers Discussed So Far

## Paper 1 – Indonesian Electric Motorcycle Battery Monitoring

Primary category: Electric Vehicle Battery Monitoring. Secondary category: IoT-Based Battery Monitoring.

The paper develops a real-time electric motorcycle battery monitoring prototype using ESP32, PZEM-017 for electrical measurements and PT100/MAX31865 for temperature. Data is sent to a cloud platform for smartphone monitoring, with Telegram alerts when voltage or temperature exceeds set limits.

Reported performance: temperature reading success of 98.15% and voltage accuracy of 99.78%.

Important limitation: the abstract describes real-time condition monitoring and threshold-based warning, rather than SOH/RUL/degradation prediction or ML-based failure forecasting.

## Paper 2 – Electric Motorcycle Battery Remaining-Life Prediction

Primary category: Battery Health Estimation & Prediction. Secondary category: Electric Vehicle Battery Monitoring.

The paper uses a Hybrid Energy Storage System (HESS) with a synthetic inductor algorithm and a real-time battery lifecycle predictor based on voltage and current measurements from real electric-motorcycle load profiles.

The proposed HESS reportedly reduced battery degradation by 52%, and the lifecycle prediction method was validated experimentally.

Important limitation: the work focuses on an electric motorcycle/HESS architecture and does not appear from the abstract to provide the proposed project's combination of conventional 12V lead-acid battery monitoring, IoT rider interface and preventive-maintenance/failure prediction.

# 9. Current Research-Gap Hypothesis

Existing literature is strong in individual areas: EV/Li-ion battery monitoring, generic lead-acid monitoring, automotive battery diagnostics, battery SOH/RUL prediction, and IoT monitoring. The project should therefore not claim that battery monitoring is new.

The research gap to investigate is the combination of a low-cost, continuously installed, IoT-enabled predictive battery health/failure monitoring system specifically designed for conventional 12V lead-acid batteries used in petrol two-wheelers.

This is a hypothesis until the literature, commercial products and patents have been systematically checked.

# 10. Recommended Survey Target

15–20 research papers

3–5 review/survey papers

5–10 commercial products

3–5 relevant patents, if available

For every paper, focus on what is measured, what is predicted, the battery/application, the algorithm, the hardware/communication architecture, and especially the limitations. The limitations across multiple papers should be used to establish the final research gap.